	[image: image1.wmf]


	[image: image2.png]





[image: image3.png]



	Emerson.com 
	Contact Us 
	Careers 

	Home
Products
Industries
News & Events
Customer Info
About us
Where To Buy
[image: image4.png]



[image: image5.png]





	


[image: image6.jpg]



B-Series Degreasers

Emerson Industrial Automation > Branson > Products > Industrial and Bransonic Cleaners > Industrial Ultrasonic Cleaning > Solvent Systems > B Series Degreasers 

	Industrial Ultrasonic Cleaning


	
	Solvent Systems
B Series Degreasers
SRS Stills
VDX 452R


	
	The Branson B-Series Ultrasonic Vapor Degreaser is the latest in a series of environmentally sound, cost effective precision degreasers.

[image: image7.png]


Equipment 

Applications 

Literature and Customer Info 

Theory 

Features:
· Freeboard ratio of greater than 120% to eliminate diffusion losses 

· Refrigerated primary condensing coil to condense the bulk of the solvent vapors 

· Refrigerated sub-zero freeboard condenser to provide a cold air barrier minimizing fugitive losses 

· Sliding cover to eliminate drafts and help seal the unit during periods of inactivity. 

· Offset boiling sump with liquid level control (except on B452R) 

· Bag filter to minimize solvent loss during maintenance 

· Cooled internal water separator to reduce losses 

· Stainless plumbing for solvent-wetted areas 

Options:
PLC-controlled vertical & dual axis lift (B452 & B950) 

Split power cover 

TDR interface Desiccant dryer 

460 V (B1950 & B3550) 

Stainless steel mesh baskets 

Water cooled compressors for clean room installations 

High frequency ultrasonics - 80 to 170 kHz 

Specifications:
  

Model B252R

Model B352R

Model B452R

Model B950R

Model B1950R

Model B3550R

Working dimensions

8" x 10" x 6"D

8" x 10" x 6"D

10" x 12" x 8"D 

12" x 16" x 12"D 

16" x 20" x 14.5"D 

20" x 24" x 17"D

Watts of Immersion Heat

1200 

1200

2000 

4000 

8000 

14000

Distillation rate** 

5.5 gph

5.5 gph

7 gph

14 gph

28 gph

49 gph

Load capacity** 

80 lbs.steel/hr

80 lbs.steel/hr

200lbs.steel/hr

400lbs.steel/hr

800lbs.steel/hr

1400lbs.steel/hr

Vapor zone

10.5" L x 12.5" W x 6"D

23.8"L x 13.3"W x 7"D

28.7"L X 15.2"W X 8.5"D

32.7"L X 19.2"W X 12"D

40.7"L X 23.2"W X 14.5"D

48.7"L X 27.2"W X 17"D

Solvent capacity

4.8 gallons

5.1 gallons

9.2 gallons

26 gallons

50 gallons

82 gallons 

Overall

22"L x 22"W x 40" H

39.7"L x 30.9"W x 45"H

54.7"L x 28.5"W x 46"H 

64.7"L x 32.5"W x 48.7"H 

89.5"L x 36.5 "W x 60"H 

110"L x 40.5"W x 68"H

Input power

208/230V, 60 Hz,
1 ph, 16

208/230V, 1 phase, 50-60Hz, 20A

208 or 230V 3phase 27A

208 or 230V 3phase 39A

208 or 230V 3phase 62A or 460, 3phase 33A option

208 or 230V 3phase 81A or 460, 3phase 42A option

 

**Based on HCFC, HFE, and HFC solvents. Specifications subject to change without notice 




_1454761240.unknown

